TWO GALLANTS Technical Rider 2013 DISREGARD PREVIOUS VERSIONS

FOH Moses Mayo Mosquito666@gmail.com 512 468 7807 TM/Band Contact Georg Geuder Cell: +49 (0)17 3565 4591 Georg.geuder@gmx.de Artist Management: Dan Kasin / Another Planet Management Office: +1 510 348 3010 Cell: +1 (415) 710 2534 email dan@anotherplanetent.com

GENERAL

- FOH, monitor and lighting engineers must be available from the specified "Get In" time. Engineers should be familar with the in-house sound and lighting systems.

- Pls have Lights, PA, Monitors and Mics set up and cabled for specified "Get In" time.

- 2GS' FOH engineer must have access to all amps, processors, crossovers, effects, EQs and especially any limiters applied to the house and monitor systems in order to "tune the room" to his specifications

- All 2GS channels and settings must be dedicated for their performance and not be altered between soundcheck and performance

- The FOH area should be unobstructed and positioned in the middle of the room

FRONT OF HOUSE

- Sound system with enough amp, speaker and sub power to address the full legal capacity of the venue with distortion-free reproduction of up to 120 dB RMS.

- High quality FOH-desk:

→ Preferred analogue desks: Midas Heritage Series, Midas XL Series or similar.

→ Digital desks are possible, preferred desks: Digidesign, PM5D, M7CL or similar

- 2 parametic 4 Band EQs. Inserts on Channel, Groups,  Master und Aux

- 10(8) Auxs (Pre/Post), at least 4 Subgroups, enough Stereo Returns for all needed effects

- 1 Multieffect (Mono In, Stereo Out) TC M2000/5000  or similar

- 2 Reverb (Mono In/Stereo Out) Lexicon PCM, SPX 900 or similar

- 1 Tap Delay (Mono In/Stereo Out) TC D2 preferred

- 6x Compressor, DBX 160 preferred, BSS/ KLARK/ or similar

- 6x Gate, Drawmer 201 BSS DPR 502 or similar

- Front EQ: 2x 31 Band 1/3 Octave EQ, EQ please in Master-Insert. Klark Teknik DN 360 or similar

- Monitor EQ’s: 6x 31 Band EQ. EQ's in Inserts please

MONITORS

2GS will not be travelling with a monitor engineer, in case of a separate monitor mix, a monitor engineer must be provided. 2GS will require:

- five (5) mixes on stage including side-fills for larger shows / festivals

- five (5) bi-amped wedges (please no drum-fill) with either 1x15” and 2” or 2x12” and 2” speakers.

DRUMRISER

- 2GS will require 1-2 risers of 3m X 2m or 8x8 in the US. → 12 inch legs preferred.  Riser must have wheels, wheels must have brakes. Primarily for festivals, we will advance if needed in clubs.

BACKDROP

- Two Gallants carry a12X20 Flame Proof back drop with ties, pipe pocket.  Please advise on mounting options .

TWO GALLANTS – Stage Plot & Input List